Project Number: 2026-Hydrogen-Based Ironmaking-Integrated-01-01

International Joint R&D of Metallic Materials

Development of Hydrogen-Reduced Iron Production Technology Achieving a Reduction Degree of 85% or Higher Based on a Hydrogen-Based Fluidized Reduction Reactor Using Iron Ore Fines

1. Project Information

Agreement period, R&D Expenses, and Implementation Structure:

• Within 57 months (Development period of Year 1: 9 months, Year 2 to 6: 12 months each).

- Phase 1: within 33 months, Phase 2: within 24 months

• R&D expenses: Within KRW 11 billion in 2026 (within total government contribution of KRW 60.5 billion).

• Lead Organization: Company (Development of Innovative Product)

• Government Technology Fee Payment Obligation: Applicable

• Each R&D project shall actively cooperate with requests from the Korean-Type Hydrogen-Based Ironmaking Demonstration Support Group to ensure smooth project operation and to analyze carbon emission reduction effects.

2. Concept/Objective

Concept:

• The hydrogen fluidized reduction reactor for iron ore reduction consists of four reaction stages. In each reactor, fluidized fine iron ore directly contacts the reducing gas and undergoes reduction, thereby producing hydrogen-reduced iron with a high reduction degree.

- Since hydrogen has lower density and viscosity than conventional reducing gases, it is unfavorable for fluidizing fine iron ore. Accordingly, the fluidized bed shall be designed and operated in consideration of the properties of hydrogen.

- To ensure that the hydrogen-reduced iron produced in the fluidized reduction reactor can be transferred to the electric melting furnace and melted with optimal efficiency, the reduction degree of the hydrogen-reduced iron shall be secured at 85% or higher.

Definition:

• Development of technology for optimizing a hydrogen fluidized reduction reactor and for producing hydrogen-reduced iron by reducing fine iron ore with supplied reducing gas to achieve a high reduction degree of 85% or higher.

Final Objective:

• Development of demonstration technology for producing hydrogen-reduced iron with a reduction degree of 85% or higher through the operation of a hydrogen fluidized reduction reactor.

- Development of engineering technology for a four-stage hydrogen fluidized reduction reactor.

- Development of process design and control technologies to secure the reduction degree of hydrogen-reduced iron and to reduce unit process consumption.

- Securing technologies for producing hydrogen-reduced iron with a high reduction degree and for reducing adhesion through the identification of iron ore types suitable for hydrogen reduction and the derivation of optimal blending ratios.

Quantitative Targets:

| No. | Key Technology / Product Performance Indicator |  | Unit | Target | Best Domestic Level | World’s Best Level (Country, Company/Organization) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Hydrogen Fluidized Reduction Reactor Demonstration Equipment |  |  |  |  |
| 1 |  | Completion of detailed design drawings | cases | 1 | - | - |
| 2 |  | Construction progress rate | % | 100 | - | - |
| 3 |  | Continuous operation time | hr | ≥300 | - | - |
| 4 | Reducing gas specific consumption1) |  | Nm3/t-ore | ≤2,500 | - | - |
| 5 | Charging rate of fine iron ore and auxiliary materials2) |  | t/hr | ≥60 | - | - |
| 6 | Circumferential temperature deviation of the fluidized bed3) |  | ℃ | ≤10 | - | - |
| 7 | Reduction degree of hydrogen-reduced iron4) |  | % | ≥85 | - | - |

1) Ratio of the reducing gas volume to the amount of fine iron ore charged into the hydrogen fluidized reduction reactor

2) Amount of fine iron ore and auxiliary materials charged into the hydrogen fluidized reduction reactor

3) Temperature deviation measured at four locations at 90° intervals in the circumferential direction at specific positions on the reactor distributor plate, used to assess the formation of a uniform internal fluidized bed

4) Ratio of the amount of oxygen removed from iron ore after reduction to the total amount of oxygen removable through reduction

Details of development:

• Verification of the basic design of the hydrogen fluidized reduction reactor

- Verification of design parameters, including critical superficial velocity and flow rate for fluidization of fine iron ore, cyclone dust collection efficiency, and reactor-specific temperature conditions required to secure the reduction degree.

- Development of a CFD (Computational Fluid Dynamics)-based fluidized reduction process simulator

- Flow analysis of zones with concentrated refractory wear within the hydrogen fluidized reduction reactor

• Detailed design and construction of hydrogen fluidized reduction reactor demonstration equipment

- Detailed design of demonstration equipment based on the results of design verification

- Construction of hydrogen fluidized reduction reactor demonstration equipment and integration testing with upstream and downstream processes

- Establishment of standard operating and management manuals for normal and abnormal operation of the hydrogen fluidized reduction reactor

• Stabilization of demonstration operation through integration of upstream and downstream processes of the hydrogen fluidized reduction reactor

- Measurement of gas composition through reactor-specific gas sampling

- Analysis of the reduction degree through sampling of hydrogen-reduced iron

- Evaluation of fluidized bed uniformity through radial temperature measurements within the hydrogen fluidized reduction reactor

- Verification of charged amounts of iron ore and auxiliary materials supplied to the hydrogen fluidized reduction reactor, hydrogen-reduced iron production volume, and related parameters

• Optimization of reactor-specific fluidized reduction conditions to improve fluidized reduction efficiency

- Identification of optimal iron ore types for hydrogen-based ironmaking and derivation of appropriate blending ratios

- Development of a reduction degree prediction model according to hydrogen fluidized reduction conditions

- Optimization of reducing gas and feedstock supply control for the hydrogen fluidized reduction reactor

- Development of technologies to reduce adhesion of reduced iron within the process

RL Core Technology Elements (CTEs):

| No. | Core Technology Element | Final TRL | Production Level or Deliverable | Test and Evaluation Environment |
| --- | --- | --- | --- | --- |
| 1 | Hydrogen fluidized reduction reactor demonstration equipment Detailed design drawings | 7 | Detailed design drawings | Self-evaluation |
| 2 | Hydrogen fluidized reduction reactor demonstration equipment | 7 | Demonstration equipment | Self-evaluation |
| 3 | Hydrogen fluidized reduction reactor demonstration technology | 7 | Official test report | Accredited certification evaluation |

Security Classification and Safety Review:

• As there are risks of accidents such as fire, explosion, and asphyxiation during the construction and operation of the demonstration equipment, this project shall be pursued as a safety management–type project.

- During construction of the demonstration equipment, there are risks of accidents such as falls from height, collisions or entrapment between construction machinery and workers, and collapse of scaffolding and temporary structures.

- During operation of the demonstration equipment (continuous operation), there are risks of accidents such as fire and explosion caused by hydrogen leakage.

3. Agency Contacts

For questions relating to request for proposal contact:

Evaluation Schedule & Procedures : Steel & Ceramic Division / +82-53-718-8464 / judong1226@keit.re.kr

Project Proposal (Planning Intent) : Metal Materials PD / +82-53-718-8369 / babeasty@keit.re.kr

The English documents provided by the KEIT are meant to be used as supplementary resources to facilitate comprehension of research and development projects. It is essential to understand that these documents do not possess any legal or official authority and should be exclusively utilized for informational purposes.